# Run the player as an app or in the browser on a Qbic panel

A Qbic panel can show the player two ways, and they are not equivalent. What the installed Offision app adds over the panel's own browser — the built-in card reader, uploaded video, the label printer — and how to tell which one a panel is running.

2026-08-29   ·   https://offision.com/help/en/devices/install-the-app-on-a-qbic-panel/

A Qbic panel runs Android, so it can show the Offision player either as an
installed app or in its own browser, pointed at the player address. Both
pair with the same six-character code, both show the same screen, and a panel
set up either way looks finished.

That is the problem. The two are not equivalent, and the differences are
silent: nothing on the panel says which one it is running, and the features it
is missing do not report an error. They simply never happen — an uploaded video
shows its poster image and never starts, a tapped card does nothing.

Install the app unless something stops you. The browser is the fallback, not
the default.

## What changes

|  | In the app | In the browser |
| --- | --- | --- |
| Uploaded video — a screen saver clip or a Video widget | Downloaded once, then plays from the panel's own storage | Never plays. The poster image stays up |
| YouTube | Plays, needs internet | Plays, needs internet |
| The built-in card reader | Reads a card tapped on the panel's face | Not reachable. A USB reader is the only way to tap |
| Label printer | Prints visitor badges — connect one | No printing |
| Light bar | Seven colours | Seven colours |
| Occupancy sensor | Reported | Reported |
| Remote reboot | From the console | From the console |

Switching to the user app is in neither column: on a Qbic panel that button
is hidden whichever way the player is running.

The bottom half of that table is why the browser is a reasonable fallback rather
than a broken one. Qbic builds a local interface into the panel, and Offision
uses it either way — so the light bar, the sensor and the remote reboot all work
in the browser. What the browser cannot do is anything needing a place to put
a file or a connection to hardware Android owns.

## Why video is the one that surprises people

Video on a screen is never streamed. The player downloads the file to the
screen's own storage first and then plays the local copy, which is what lets a
lobby video keep running through a network outage. A panel in browser mode has
nowhere to put that file, so it shows the poster image and never starts —
whatever the format is.

Nothing about this depends on the file. Re-exporting the video will not fix it,
and neither will a smaller one. It is the container the player is running in.

A YouTube item is different, because it is a live stream rather than a
download, and it plays either way. On a panel that has to stay in browser mode,
that is the workaround: put the clip on YouTube.

The playlist itself is in
Play your company video on idle room panels.

## Which one is this panel running

The console cannot tell you. A Qbic panel reports the same Container value
whichever way it is running, so the status page is not the place to look.

Test it at the panel. Tap a staff card on its face: if nothing happens on a
panel ordered with the reader, it is running the browser.

Where in Offision: admin app → device/booking-panel/device (Booking panel)

On a panel with no built-in reader, use video instead. Put an uploaded clip on
it and watch: the app plays it, the browser sits on the poster image.

## Installing the app

The app is an ordinary Android package, installed at the panel from a USB
drive — the download, the device settings it needs, and the field that decides
what the panel launches afterwards are all in
Install the Offision app on a Qbic device.

For more than a handful of panels, install it through Qbic's own device
management rather than by hand — the same tool that handles firmware. Offision
does not push apps to panels.

After installing, pair again:
A panel that was already paired in browser mode does not carry that over. The
app starts fresh and shows a new pairing code, so claim it again on its
existing row rather than adding a second device.

## What this does not change

- It does not change what the panel can do. The light bar, the sensor and
the card reader are the hardware's, and a panel that never had a built-in
reader does not gain one by installing the app.

- It does not change the licence. A panel needs Hardware either way.

- It does not change the room. The room is chosen when the panel is paired,
not by how the player is running.

- It is not how a panel is updated. The player updates itself; the app
around it is updated like any other Android app on the panel.