# Put visitors on a secured WPA-Enterprise network

Create a Visitor WiFi entry for Cisco Meraki, UniFi, TP-Link, Ruckus or any standard access point, point the access point at Offision's RADIUS server, and have visitors join with the username and password on their badge.

2026-08-20   ·   https://offision.com/help/en/visiting/set-up-radius-visitor-wifi/

Before you start:
You need admin access to your WiFi controller, a Visitor WiFi Integration
license for the network, and access points that can reach Offision's RADIUS
server — UDP port 1812, or RadSec over TCP 2083.

Done looks like this: a test visitor checks in, joins your guest SSID with the
username and password on their badge, and is refused after checking out.

## 1. Add the network entry

Open Visitor WiFi, choose Add, and pick the brand that matches your
controller from the menu — Cisco, Ruckus, TP-Link, Unifi, or
Generic for any standard RADIUS access point. The brand cannot be changed
after the entry is created.

Where in Offision: admin app → visiting/visitor-wifi (Visitor WiFi)

The Visitor WiFi page — one row per guest network.

The Marketplace also carries a tile per vendor under Visitor WiFi; a tile
opens this same list with the brand already chosen.

Note:
Picking a brand creates the entry immediately, so its RADIUS values exist from
the start. Closing the dialog without saving leaves the entry in the list —
rename it, or delete it.

## 2. Fill in the Basic page

Name the entry, decide the sign-in rules — Username type and
Password type — and, if this network is for one site only, link its
buildings.

A RADIUS entry's Basic page — the sign-in rules, and no network-name field.

Every field is explained in
the settings reference. There is no
network-name field on this page: a secured network is named on your controller,
and its security comes from the per-visitor sign-in, not a shared password.

## 3. Copy the RADIUS values into your controller

Open RADIUS settings, the second page of the dialog. Everything on it is
generated for you: Host IP or FQDN, Auth port, Secret, and the
NAS identifier.

The RADIUS settings page — values to copy into the controller.

Two brands differ:

- Unifi authenticates by access point instead: enter your access points'
MAC addresses, one per line, rather than a NAS identifier.

- Cisco shows the same values arranged as a step-by-step Meraki guide.

## 4. Create the SSID on your controller

On the controller, create a WPA-Enterprise (802.1X) guest SSID pointing at the
values from step 3. Offision's RADIUS server answers PAP, EAP-TTLS/PAP and
PEAP/MSCHAPv2 — the methods phones and laptops use out of the box.

## 5. Check it worked

Invite yourself, check in at reception, and join the SSID with the username and
password printed on your badge. Then check out — and confirm the same details
are now refused. Both results matter: the second one is the security story.

## When it goes wrong

| What you see | Usual cause |
| --- | --- |
| Sign-in refused for a real visitor | They have not checked in yet, or their visit period has ended |
| Every sign-in refused | The entry has no Visitor WiFi Integration license, or the controller's shared secret no longer matches — regenerating the Secret without updating the controller does this |
| The controller never reaches Offision | The NAS identifier (or a Unifi MAC address) is missing or mistyped on the access points, so requests cannot be matched to the entry |
| The badge prints no WiFi details | Expected for RADIUS brands — see what the badge can print |